| Product: | ## RTL8712(8188_8191_8192SU) USB Software Package - Linux Driver |
| --- | --- |
| Version: | 2.6.0005.p4 |
| Date: | ## 2009/11/12 |

Release History:

| V1.0.0001 – 2009/01/21 | - First Released - only supports USB interface - add features about 802.11n draft 2.0 - RX-AMSDU (4K, 8K) - currently only supports 20MHz BW |
| --- | --- |
| V2.0.0001 – 2009/4/27 | - WiFi 11n-Certificated - Supported 20/40 MHz BW Switching - Supported Fedora 10 |
| V2.6.0001 – 2009/9/30 | - Add Rx_BH - Add Tx_BH - Fixed system hung up issue when cancel rx timer. |
| V2.6.0005 – 2009/10/13 | - put usb rx aggregation to the rx_bh - support Rx TCP_CSUM_OFFLOAD - fixed ping issue on DMP Platform with D-LINK DIR655 AP - To report encryption_cap. When wpa_supplicant & Network Management require |
| V2.6.0005 – 2009/11/12 | - fixed ad-hoc issue when using Network Manager - update fw to fixed long run testing issue |
| V2.6.0005.p1 – 2009/11/12 | 1. Fixed the power saving issue. |
| V2.6.0005.p2 – 2009/11/12 | 1. support high power mode |
| V2.6.0005.p3 – 2009/11/12 | - Added a script to clean the r8192s_usb driver for avoiding the conflict that 8712u and r8192s_usb drivers are running at the same time. - Updated the thread_enter function for some SDIO bugs. |
| V2.6.0005.p4 – 2009/11/12 | - Added two VID/PIDs into supporting list. - The driver will disable the 11n mode if the attaching device is with these two new VID/PIDs. |

Description:

Realtek provides the solution for WLAN in hardware and software. The software package could be adopted in Realtek demo-board in the Linux platform.

Software Version:

RTL8712 USB Linux Driver Version: v2.6.0005.20091112.p4

Firmware Version:

Firmware is embedded in the RTL8712 USB driver.

The firmware version of this release is SVN169

Software Package Included:

a. Readme.txt

b. Release Notes

c. Linux driver

d. wpa_supplicant - wpa_supplicant-0.6.9_wps_patch.tar.gz

e. document - quick_installation_guide.ppt

Hardware Supported:

Realtek RTL8712/8188/8191/8192SU USB demo board:

- WIFI – SoC, including a CPU, one 802.11b/g/n MAC/Baseband/RF controller with PA for the USB interface.

Platform Supported:

- USB : LINUX (kernel 2.6.18 ~ 2.6.29)

Software Package Feature:

- IEEE 802.11 b/g/n WiFi compliant

- 802.1x, WEP, WPA TKIP and WPA2 AES/Mixed mode for PSK and TLS (Radius)

- Wireless Auto-channel selection

- WPS - PIN and PBC Methods

- IEEE 802.11b/g/n client mode

  - Support Ad-hoc and infrastructure mode with roaming capability

  - Support wireless security for WEP, WPA TKIP and WPA2 AES PSK

  - Support site survey scan and manual connect

  - Support WPA/WPA2 TLS client

To-Do List:

- To Add Soft AP Mode